| Risk Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation / Work activity being assessed: |  |  |  |  |  |  |  |  |  |
|  |  | Routine |  |  | Non-routine |  |  |  |  |
| Generated by: Vessel (record the name) |  |  |  |  |  | Office |  |  |  |
| Code number (to be assigned by the Office): |  |  |  |  |  | Version: |  | Date: |  |

| FREQUENCY CATEGORY |  |  |  | CONSEQUENCE CATEGORY |  |  |  |  |  | RISK MATRIX |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Frequent - Possibility of repeated incidents | More often than once per voyage |  | 4 | Human losses / fatalities | Major pollution / Full scale response | Excessive / high-cost damage > $1000000 | Major national & international impact |  |  |  | FREQUENCY |  |  |  |  |
| 4 | Probable - Possibility of isolated incidents | Once per year |  | 3 | Serious injury to personnel | Moderate pollution/ Significant resources commitment | Moderate cost or damage ($100000 –$1000000) | Considerable impact |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 1 | 2 | 3 | 4 | 5 |
| 3 | Occasional- Possibility of occurring sometime | Once per 5 years |  | 2 | Number of minor injuries / Medical treatment for personnel | Little pollution / Limited response of short duration | Little cost or damage ($10000 – $100000) | Slight impact |  | CONSEQUENCE | 1 | L(1) | L(2) | L(3) | M(4) | M(5) |
|  |  |  |  |  |  |  |  |  |  |  | 2 | L(2) | M(4) | M(6) | M(8) | H(10) |
| 2 | Remote- Not likely to occur | Once per 10 years |  | 1 | Few minor injuries | Minimum pollution / Little or no response needed | Minimum cost / damage < $10000 | Zero impact |  |  | 3 | L(3) | M(6) | M(9) | H(12) | H(15) |
| 1 | Very unlikely- Practically impossible | Once per 30 years or more |  |  |  |  |  |  |  |  | 4 | M(4) | M(8) | H(12) | H(16) | H(20) |
|  |  |  |  |  |  |  |  |  |  | High=Intolerable Risk Medium=Tolerable Risk Low=Negligible Risk |  |  |  |  |  |  |

| Risk Assessment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Risk Identification |  | Risk Analysis | Initial risk evaluation |  |  |
| No | Hazard | Potential hazardous event | Existing control measures | F | C | R |
| 1 | Personnel injury | Serious injury to personnel | - Vms chapter 7.3.4 Anchoring operations - OCIMF Anchoring Systems & procedures - Use of adequate PPE as per company’s PPE matrix - Well trained personnel - Continuous control by supervisor - PMS maintenance instructions - Experienced personnel on duty - Full navigational equipment/means exploitation - Careful selection of anchoring seabed - Readiness to move from anchorage - Use of navigation info publications - Collect meteo information - Masters standing orders - Continuous anchor watch. OoW should monitor ship’s position & ensure that the ship is not drugging towards land or another anchored ship. - Use of all detection equipment - Readiness to change anchorage. The anchor should not be dropped from the hawse-pipe but should be lowered to a suitable position & let go from that position | 4 | 3 | 12 |
| 2 | Loss of anchor | Moderate cost or damage |  | 5 | 3 | 15 |
| 3 | Anchor dragging | Little cost or damage |  | 4 | 2 | 8 |
| 4 | Inadequate seabed quality | Little cost or damage |  | 5 | 2 | 10 |
| 5 | Deteriorization of weather / visibility condition | Moderate cost or damage |  | 3 | 3 | 9 |
| 6 | Windlass failure / damage | Moderate cost or damage |  | 4 | 3 | 12 |
| 7 | Dense navigation traffic | Moderate cost or damage |  | 3 | 3 | 9 |
| 8 | Lack of knowledge of drop position & exact swinging cycle | Moderate cost or damage | Fix the anchor drop position and ascertain the ship’s swinging circle, based on the length of the cable in use | 1 | 3 | 3 |
| 9 | Lack of knowledge of ship position | Moderate cost or damage | Landmarks and transits should be selected for ease of monitoring the position of the ship as it lies at anchor | 1 | 3 | 3 |
| 10 | Lack of visible warning signs to passing ships | Moderate cost or damage | Appropriate light and shape signals should be exhibited according to COLREGS and any local regulations | 1 | 3 | 3 |
| 11 | Uncertainty on future developments | Moderate cost or damage | The Master should be immediately notified if the ship drags her anchor and if sea conditions or visibility deteriorate | 1 | 3 | 3 |
| 12 | Anchors may be let-go at inappropriate moments due to the wrong message being received on the portable radio | Moderate cost or damage | Establish constant communication with bridge. All instructions should be passed at a VHF channel used by the ship only | 1 | 3 | 3 |
| 13 | Anchoring equipment failure under load | Moderate cost or damage | Windlass should be tested prior its use. Anchor, chains, swivels, keys, stoppers, guars etc. should be inspected prior use | 1 | 3 | 3 |
| 14 | Exceed SWL | Moderate cost or damage | Remember windlass is designed to pay out before exceeding SWL. | 1 | 3 | 3 |
| 15 | Head & eye injury from dust & debris thrown form the anchor | Serious injury to personnel | Seafarers engaged in operating the brake, and others in the vicinity should wear goggles and safety helmets | 1 | 3 | 3 |
| 16 | Injury of personnel stowing the anchor chain into the locker | Serious injury to personnel | Stand in a protected position and keep in constant communication with the windlass operator | 1 | 3 | 3 |
| 17 | Accidental anchor release due to anchor windlass brake failure | Moderate cost or damage | Anchors housed and not required should be properly secured to guard against accidents or damage | 1 | 3 | 3 |
| 18 | Slip & fall on wet deck or loose hand grip | Serious injury to personnel | Anti-slip paint in way of mooring equipment | 3 | 3 | 9 |
| 19 | Lack of knowledge of drop position & exact swinging cycle | Moderate cost or damage | Fix the anchor drop position and ascertain the ship’s swinging circle, based on the length of the cable in use | 1 | 3 | 3 |
| Note F: Frequency, C: Consequence, R: Risk |  |  |  |  |  |  |

| Risk Treatment |  |  |  |  |  |  | Residual risk evaluation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hazard No. | Additional Risk Control Measures |  |  | Responsible |  | Action Timeline | F |  | C |  | R |  |
| 1 |  | Consider to change anchoring position | C/O |  | During task |  |  |  |  |  |  |  |
| 2 |  | Spares availability – use of the other windlass | C/O |  | Before task |  |  |  |  |  |  |  |
| 3 |  | Prepare to leave anchorage | C/O |  | Before task |  |  |  |  |  |  |  |
| 4 |  | Prepare to leave anchorage | C/O |  | Before task |  |  |  |  |  |  |  |
| 5 |  | Consider to change anchoring position | C/O |  | During task |  |  |  |  |  |  |  |
| 6 |  | Spares availability – use of the other windlass | C/O |  | Before task |  |  |  |  |  |  |  |
| 7 |  | Prepare to leave anchorage | C/O |  | Before task |  |  |  |  |  |  |  |
| 18 |  | Ensure that personnel wear Safety shoes, leather gloves | C/O / Safety Officer |  | Before work done |  |  |  |  |  |  |  |
| Note F: Frequency, C: Consequence, R: Risk |  |  |  |  |  |  |  |  |  |  |  |  |

ALTERNATIVE WAYS TO CONDUCT THE WORK (a new RA should be conducted for the alternative way decided)

| Are any amendments to UMMS required (related to the above Additional Risk Control Measures)? |  |  |  |
| --- | --- | --- | --- |
| Yes |  | Describe: |  |
| No |  |  |  |

| Contingency plans (to facilitate safe management and recovery of the situation in case of any unplanned occurrences) |  |
| --- | --- |
| Emergency case | Contingency Plans |

| Risk Management Team |  |  |  |
| --- | --- | --- | --- |
| Name | Rank / Title | Signature | Date |
| Remarks: |  |  |  |

| Risk Assessment reviewed/approved by: |  |  |  |
| --- | --- | --- | --- |
| Name | Rank / Title | Signature | Date |
| Remarks: |  |  |  |